Rancho Technology, Inc.

8632 Archibald Ave. Suite 109

Rancho Cucamonga, CA  91730

Dear Valued Customer,

FMTFLOPT is a Floptical diskette formatter that will allow you to format

21Mb, 1.44Mb and 720Kb floppy diskettes in a Floptical drive. If you have

a Floptical drive in the system, and you will be formatting 21Mb diskettes,

you must load FMTFLOPT.EXE first.

To load FMTFLOPT, type the following at your DOS PROMPT:

FMTFLOPT <Enter>

This will set the default parameters and will enable you to use DOS'

FORMAT.COM program to format Floptical 21Mb diskettes.  To format a 21Mb

diskette, place the diskette into the drive, then enter the following at

your DOS PROMPT:

FORMAT d: <Enter>

Where d: is the drive letter for the Floptical drive.  Then follow the

instructions on your screen and your diskette will soon be formatted.

FOR ADVANCED USERS

------------------

FMTFLOPT has four options that may be set at any time.  The following

describes those options.

FMTFLOPT [/?] [/D[+|-]] [/L[+|-]] [/T[+|-]] [/I[<interleave>]]

/?  Displays a help screen similar to this one

/D  Reports/Changes the format type of the disk (bootable/not bootable)

/L  Reports/Changes the forced low level format flag (on/off)

/T  Reports/Changes the translation status of FMTFLOPT (on/off)

/I  Reports/Changes the interleave for low level format

<interleave> is the new interleave value to set for low level format [0..7]

1.  At the DOS prompt, load FMTFLOPT by typing

FMTFLOPT <Enter>

This will initialize the program parameters to their default settings.

You may also specify different default settings as shown in the

command line format above.  For example, to make the default low level

interleave 2:1, use the following command line:

FMTFLOPT /I2 <Enter>

2.  You may change the status of the parameters either at load time or

any time thereafter.  To do this, just run FMTFLOPT with the proper

command line settings.

For example, if you wish to low-level format your diskette, type

FMTFLOPT /L+ <Enter>, and the low level format flag will be ON.  If

you wish to make your diskettes non-bootable, type FMTFLOPT /D- <Enter>,

and your diskettes will not be bootable--and so forth.  Changes to any

one of these parameters will be reflected in the list of the parameter

settings.  Type FMTFLOPT <Enter> to make sure the parameters are set

as required.

3.  After you have set the parameters accordingly, you use DOS' FORMAT

program to format all three densities of 3.5" diskettes.

APPLICATION NOTES:

1.  Do not use Quick Format on 21Mb diskettes.

2.  If you wish to format a 21Mb diskette, you must have it in the drive

before you run FORMAT.

3.  When you run FORMAT to format a 21Mb disk, you may continue formatting

21Mb diskettes, but not 1.44Mb or 720K diskettes.  You must terminate

the FORMAT program and run FORMAT again if you wish to format

another type of media.

4.  /L   Low-level Format

a.  If FMTFLOPT notices that the diskette is not low level formatted,

it will automatically force a low-level format.

b.  When running DOS 5.x, you must use the FORMAT /U switch in unison

with the FMTFLOPT /L+ switch in order to low level format a disk.

c.  The low-level format prompt is not displayed under

Windows 3.x or Windows 3.x DOS prompt.

5.  /T  Translation

If you experience any problems with your applications,

type FMTFLOPT /T- <Enter> to turn the translation status

OFF and see if the problem goes away.

6.  /D  Format Disk Type

Never use the switch (/s) to make a system diskette when

you have set the FMTFLOPT /D- switch.  DOS will allow this to

happen, but any disks formatted in this manner will not be

bootable.

7.  FMTFLOPT and SMARTDRV.EXE

DOS' SMARTDRV has some problems when loading with FMTFLOPT.  To

alleviate all problems, we suggest that you load SMARTDRV first,

then load FMTFLOPT.

###################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################